| EWAH-SM-03 ¦ MESH FLOW SHORT SET PERFORATED SUMMER SET — ABOVE-THE-KNEE VERSION | E W A H SCULPT · FLOW · FREEDOM |
| --- | --- |

Design reference board — separate perforated top and lined skirt/skort.

# 1. Style Overview

| Field | Specification |
| --- | --- |
| Style code | EWAH-SM-03 |
| Style name | Mesh Flow Short Set |
| Category | Two-piece perforated top and skirt/skort |
| Development status | Tech Pack Draft v0.1 |
| Fit intent | Relaxed top with shaped side panels; adjustable-waist skirt above knee with modest opaque inner short. |
| Primary use | Travel, resort, walking, light training and gym-to-life. |
| Ownership | Original EWAH design. Patterns, grading and technical files to remain EWAH property subject to contract. |
| Critical note | All measurements are preliminary development specifications and must be validated by a qualified pattern maker through physical fit samples. |

# 2. Design Architecture & Technical Callouts

| Ref | Feature | Requirement | Non-negotiable |
| --- | --- | --- | --- |
| 01 | Relaxed perforated top | Sleeveless, softly shaped, hip-length; opaque fitted lining option. | Relaxed—not boxy or shapeless. |
| 02 | Top side sculpting | Solid or denser-knit contour panels at waist. | Preserve EWAH seam language. |
| 03 | Skirt outer layer | Open performance mesh, above-knee, slight A-line/column hybrid. | Must not cling or balloon. |
| 04 | Adjustable waist | Elastic waistband with external drawcord. | No bulky gathered waist. |
| 05 | Side slits | Open side panels/slits for mobility. | Opaque inner short remains covered. |
| 06 | Inner short | Attached or modular opaque short, 5 in inseam. | No front seam; squat-safe. |
| 07 | Back identity | Vertical back branding on solid spine/panel. | Simplified mark sized for mesh. |

# 3. Bill of Materials (BOM)

| # | Component | Material / specification | Color | Supplier / code | Approval |
| --- | --- | --- | --- | --- | --- |
| 1 | Outer mesh | 92% nylon / 8% spandex open performance mesh, 150–180 GSM | Onyx | TBD | Swatch |
| 2 | Top lining | 75% nylon / 25% spandex jersey, 180–220 GSM | Onyx | TBD | Swatch |
| 3 | Inner short | 75% nylon / 25% spandex interlock, 230–260 GSM | Onyx | TBD | Swatch |
| 4 | Waist elastic | Soft plush-back elastic, 35–45 mm | Black | TBD | Proto |
| 5 | Drawcord | Matte round or flat elastic cord + soft tips | Black | TBD | Proto |
| 6 | Logo | Stretch transfer or embroidery trial | Tonal/gold | TBD | Strike-off |

# 4. Construction & Stitching

| Ref | Area | Construction requirement | Stitch / finish | Tolerance |
| --- | --- | --- | --- | --- |
| C1 | Top seams | Mesh and sculpting panels aligned symmetrically. | Overlock / flatlock trial | 1/8 in |
| C2 | Top neck/armhole | Soft binding, no scratch or gaping. | Self binding / coverstitch | 1/8 in |
| C3 | Skirt waistband | Distribute ease evenly; drawcord exits reinforced. | Overlock + coverstitch | 1/4 in |
| C4 | Skirt slits | Reinforced slit apex; clean narrow finish. | Coverstitch / binding | 1/8 in |
| C5 | Inner short | Secure attachment with low bulk. | Overlock + coverstitch | 1/4 in |
| C6 | Mesh hem | No waviness or stretched edge. | Narrow coverstitch / bonded | 1/8 in |

# 5. Points of Measure — Base Size M

| POM | Measurement point | Spec (in) | Tol. ± | How measured |
| --- | --- | --- | --- | --- |
| T01 | Top bust circumference | 39.0 | 0.5 | 1 in below armhole ×2 |
| T02 | Top waist circumference | 36.0 | 0.5 | At narrowest shaping point ×2 |
| T03 | Top front length | 22.0 | 0.4 | HPS to hem |
| T04 | Top armhole circumference | 18.0 | 0.3 | Along seam |
| S01 | Skirt waist relaxed | 29.0 | 0.5 | Waist edge ×2 |
| S02 | Skirt waist stretched | 40.0 | 0.75 | Comfortable max stretch ×2 |
| S03 | Skirt full hip | 43.0 | 0.5 | At fullest point ×2 |
| S04 | Skirt side length | 17.0 | 0.4 | Top waist to hem |
| S05 | Side slit height | 6.0 | 0.25 | Hem to apex |
| S06 | Inner-short inseam | 5.0 | 0.25 | Crotch to leg opening |

# 6. Preliminary Grade Rules

| Measurement | XS→S | S→M | M→L | L→XL | XL→2XL | 2XL→3XL | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Top bust | 1.5 | 1.5 | 1.5 | 2.0 | 2.0 | 2.0 | Maintain relaxed ease |
| Top waist | 1.5 | 1.5 | 1.5 | 2.0 | 2.0 | 2.0 | Do not lose sculpting |
| Skirt hip | 1.75 | 1.75 | 2.0 | 2.0 | 2.25 | 2.25 | Validate on fuller glutes |
| Skirt waist relaxed | 1.0 | 1.0 | 1.25 | 1.25 | 1.5 | 1.5 | Elastic and drawcord adjustability |
| Skirt length | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | Keep above-knee intent |
| Inner-short inseam | 0 | 0 | 0 | 0 | 0 | 0 | Fixed 5 in |

Grade rules are provisional. Final grading must preserve the waist-to-hip relationship, rear coverage, slit position, perforation map and underlayer security on every size.

# 7. Branding, Labels & Packaging

| Element | Placement | Size / method | Color | Notes |
| --- | --- | --- | --- | --- |
| Top front emblem | Left chest | 12–15 mm | Tonal/gold | Subtle |
| Top vertical lockup | Upper center back | 70–90 mm | Tonal/gold | On solid spine if needed |
| Skirt wordmark | Upper left hip | 35–50 mm wide | Tonal/gold | Avoid placement over high-stretch zone |
| Internal print | Top inside back / skirt inside waistband | Brand + size + fiber | Tonal | Heat transfer |
| Hangtag | Waist or side seam | Product story + barcode | Black/ivory/gold | Removable |

# 8. Performance & Quality-Control Tests

| Test | Method / movement | Pass criteria | Stage |
| --- | --- | --- | --- |
| Coverage | Standing, seated, forward bend | Opaque inner layers fully cover bust/seat | Proto |
| Slit mobility | Walk, lunge, stairs | No restriction; slit remains controlled | Proto |
| Waist security | 15 min movement circuit | Skirt does not slide or twist | Wear test |
| Mesh snag | Rub against smooth equipment simulation | No immediate snagging or laddering | Material |
| Recovery | 20 stretch cycles | No permanent distortion >3% | Lab |
| Wash | 5 home-laundry cycles | No hem wave, logo damage or mesh deformation | PP |

# 9. Sample Review & Revision Log

| Round | Date | Sample type | Issue / observation | Required correction | Status |
| --- | --- | --- | --- | --- | --- |
| Proto 1 |  | Development sample |  |  | Open |
| Proto 2 |  | Revised fit sample |  |  | Pending |
| Size set |  | XS–3XL |  |  | Pending |
| PP sample |  | Pre-production |  |  | Pending |

# 10. Approval Gates

☐ Design architecture approved

☐ Fabric and color approved

☐ Base-size fit approved

☐ Movement and modesty tests passed

☐ Size-set approved

☐ Pre-production sample approved