| EWAH-ST-02 ¦ AIR SCULPT SHORT DRESS MODEST SUMMER TRAINING DRESS — SHORT PERFORATED VERSION | E W A H SCULPT · FLOW · FREEDOM |
| --- | --- |

Design reference board — aesthetic intent, not final production drawing.

# 1. Style Overview

| Field | Specification |
| --- | --- |
| Style code | EWAH-ST-02 |
| Style name | Air Sculpt Short Dress |
| Category | Performance dress with attached inner short |
| Development status | Tech Pack Draft v0.1 |
| Fit intent | Sculpted through bust and waist; additional room through hips/glutes; above-knee modest outer length. |
| Primary use | Strength training, walking, travel and gym-to-life. |
| Ownership | Original EWAH design. Patterns, grading and technical files to remain EWAH property subject to contract. |
| Critical note | All measurements are preliminary development specifications and must be validated by a qualified pattern maker through physical fit samples. |

# 2. Design Architecture & Technical Callouts

| Ref | Feature | Requirement | Non-negotiable |
| --- | --- | --- | --- |
| 01 | Upper chest panel | Strategic perforated panel over opaque/supportive inner layer. | Airflow without visible cleavage or bra exposure. |
| 02 | Sculpted bodice seams | Curved front/back seams frame waist and support fuller bust. | No generic A-line block. |
| 03 | Side ventilation | Perforated side inserts follow contour line. | Must not reveal underwear or create side bulging. |
| 04 | Side movement slits | Two curved slits integrated into outer dress. | Short remains covered while standing; opens only in movement. |
| 05 | Built-in short | Attached at internal waist; 5 in inseam; right phone pocket. | No front seam; opaque; must not ride excessively. |
| 06 | Back identity | Perforated upper-back panel with vertical EWAH lockup. | Use simplified one-color mark for production. |

# 3. Bill of Materials (BOM)

| # | Component | Material / specification | Color | Supplier / code | Approval |
| --- | --- | --- | --- | --- | --- |
| 1 | Main body | 75% nylon / 25% spandex matte 4-way stretch, 220–250 GSM | Onyx | TBD | Lab dip |
| 2 | Perforated panels | Performance perforated knit, 85% nylon / 15% spandex, 150–180 GSM | Onyx | TBD | Swatch |
| 3 | Inner short | Opaque nylon/spandex interlock, 230–260 GSM | Onyx | TBD | Swatch |
| 4 | Pocket bag | Self or lightweight power mesh | Onyx | TBD | Proto |
| 5 | Logo transfer | Matte silicone or high-stretch heat transfer | Tonal / antique gold | TBD | Strike-off |
| 6 | Care label | Soft satin or printed tear-away | Black | TBD | Artwork |

# 4. Construction & Stitching

| Ref | Area | Construction requirement | Stitch / finish | Tolerance |
| --- | --- | --- | --- | --- |
| C1 | Contour seams | Join without puckering; seam line must remain smooth under stretch. | 4-thread overlock + coverstitch/topstitch as approved | 1/8 in |
| C2 | Mesh joins | Stabilize mesh-to-solid transitions; no rippling. | Narrow overlock or bonded seam trial | 1/8 in |
| C3 | Neck/armhole | Clean, soft binding; no gaping or cutting. | Self binding / bonded finish | 1/8 in |
| C4 | Side slits | Reinforce slit apex; clean inside finish. | Narrow coverstitch or bonded | 1/8 in |
| C5 | Inner short | Attach cleanly at waist; avoid bulk; phone pocket secure. | Overlock + coverstitch | 1/4 in |
| C6 | Hem | Curved hem must lie flat with no tunneling. | Twin needle / bonded hem | 1/8 in |

# 5. Points of Measure — Base Size M

| POM | Measurement point | Spec (in) | Tol. ± | How measured |
| --- | --- | --- | --- | --- |
| P01 | Bust circumference, 1 in below armhole | 38.0 | 0.5 | Garment laid flat ×2 |
| P02 | Waist circumference at narrowest seam | 31.0 | 0.5 | Garment relaxed ×2 |
| P03 | Full hip circumference | 43.0 | 0.5 | At fullest outer-dress point ×2 |
| P04 | Center back length | 33.0 | 0.4 | HPS/CB neck to hem |
| P05 | Front shortest length | 30.0 | 0.4 | HPS to shortest front hem |
| P06 | Armhole circumference | 17.5 | 0.3 | Along seam line |
| P07 | Side slit height | 9.5 | 0.25 | Hem to slit apex |
| P08 | Inner-short inseam | 5.0 | 0.25 | Crotch seam to leg opening |
| P09 | Inner-short waist relaxed | 29.0 | 0.5 | Waist edge ×2 |
| P10 | Phone pocket usable opening | 5.5 | 0.25 | Finished opening |

# 6. Preliminary Grade Rules

| Measurement | XS→S | S→M | M→L | L→XL | XL→2XL | 2XL→3XL | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bust circumference | 1.5 | 1.5 | 1.5 | 2.0 | 2.0 | 2.0 | Total circumference increment |
| Waist circumference | 1.25 | 1.25 | 1.5 | 1.5 | 2.0 | 2.0 | Protect waist-to-hip ratio |
| Full hip circumference | 1.75 | 1.75 | 2.0 | 2.0 | 2.25 | 2.25 | Validate on fuller glutes |
| Body length | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | Do not shorten larger sizes |
| Armhole circumference | 0.25 | 0.25 | 0.25 | 0.3 | 0.3 | 0.3 | Check bra coverage |
| Short inseam | 0 | 0 | 0 | 0 | 0 | 0 | Fixed 5 in unless wear test requires |

Grade rules are provisional. Final grading must preserve the waist-to-hip relationship, rear coverage, slit position, perforation map and underlayer security on every size.

# 7. Branding, Labels & Packaging

| Element | Placement | Size / method | Color | Notes |
| --- | --- | --- | --- | --- |
| Front emblem | Lower left hem or left chest | 12–16 mm high transfer | Tonal | Quiet branding |
| Vertical lockup | Upper center back within solid spine | 70–90 mm high | Tonal/gold | Must remain legible on mesh |
| Internal heat transfer | Inside upper back | Brand, size, country, fiber | Tonal | No scratchy neck label |
| Care label | Inside left side seam | Legally required content | Black/white | Confirm market requirements |
| Packaging | Recycled polybag + hangtag | Style/color/size barcode | Ivory/black/gold | No unverified sustainability claims |

# 8. Performance & Quality-Control Tests

| Test | Method / movement | Pass criteria | Stage |
| --- | --- | --- | --- |
| Squat opacity | 5 deep squats under bright light | No transparency at outer dress or inner short | Proto |
| Slit exposure | Lunge, step-up, seated position | Under-short provides full coverage; slit does not migrate | Proto |
| Neck security | Overhead reach and forward bend | No gaping or unintended exposure | Proto |
| Short ride-up | 15 min walking + squat circuit | No severe rolling or chafing | Wear test |
| Recovery | 20 stretch cycles | No permanent distortion >3% | Lab |
| Wash | 5 home-laundry cycles | No seam failure, logo cracking or mesh distortion | PP |

# 9. Sample Review & Revision Log

| Round | Date | Sample type | Issue / observation | Required correction | Status |
| --- | --- | --- | --- | --- | --- |
| Proto 1 |  | Development sample |  |  | Open |
| Proto 2 |  | Revised fit sample |  |  | Pending |
| Size set |  | XS–3XL |  |  | Pending |
| PP sample |  | Pre-production |  |  | Pending |

# 10. Approval Gates

☐ Design architecture approved

☐ Fabric and color approved

☐ Base-size fit approved

☐ Movement and modesty tests passed

☐ Size-set approved

☐ Pre-production sample approved