| EWAH-WF-01 ¦ WARRIOR FLOW DRESS-SKORT SCULPTED PERFORMANCE DRESS — ASYMMETRIC FLOW VERSION | E W A H SCULPT · FLOW · FREEDOM |
| --- | --- |

Primary design direction — use together with approved length and seam references.

# 1. Style Overview

| Field | Specification |
| --- | --- |
| Style code | EWAH-WF-01 |
| Style name | Warrior Flow Dress-Skort |
| Category | Premium asymmetric performance dress with inner short |
| Development status | Tech Pack Draft v0.1 |
| Fit intent | Secure wrap-inspired bodice, defined waist, mixed smooth and controlled-flow panels, room through hips/glutes. |
| Primary use | Strength training, studio, walking, travel and gym-to-life. |
| Ownership | Original EWAH design. Patterns, grading and technical files to remain EWAH property subject to contract. |
| Critical note | All measurements are preliminary development specifications and must be validated by a qualified pattern maker through physical fit samples. |

# 2. Design Architecture & Technical Callouts

| Ref | Feature | Requirement | Non-negotiable |
| --- | --- | --- | --- |
| 01 | Neckline | Secure wrap-inspired V or crossover illusion with internal stabilization. | No deep cleavage, gaping or literal costume closure. |
| 02 | Bodice seams | Architectural curved seams frame bust, waist and back. | Strong EWAH sculptural identity. |
| 03 | Waist structure | Clean shaped waist without bulky ties. | Must remain flat during movement. |
| 04 | Asymmetric outer panels | Smooth main panel plus selective controlled pleat/flow panel. | Not fully pleated; no excessive volume. |
| 05 | Movement openings | Dual side/asymmetric openings reveal inner short only in motion. | Standing coverage must remain modest. |
| 06 | Inner short | Attached 5 in inseam short with optional phone pocket. | Opaque, no front seam, secure. |
| 07 | Back seams/brand | Sculpted back with optional vertical lockup. | No generic racerback aesthetic. |

# 3. Bill of Materials (BOM)

| # | Component | Material / specification | Color | Supplier / code | Approval |
| --- | --- | --- | --- | --- | --- |
| 1 | Main body | 75–80% nylon / 20–25% spandex matte jersey, 230–260 GSM | Onyx | TBD | Swatch |
| 2 | Flow panel | Stretch woven or lighter jersey, 160–200 GSM | Onyx | TBD | Drape test |
| 3 | Optional pleat insert | Heat-set stretch-compatible micro-pleat | Onyx | TBD | Wash test |
| 4 | Inner short | Opaque nylon/spandex interlock, 230–270 GSM | Onyx | TBD | Swatch |
| 5 | Stabilizer | Light clear elastic / knit stay tape | Clear/black | TBD | Proto |
| 6 | Logo | High-stretch transfer / embroidery trial | Tonal/gold | TBD | Strike-off |

# 4. Construction & Stitching

| Ref | Area | Construction requirement | Stitch / finish | Tolerance |
| --- | --- | --- | --- | --- |
| C1 | Crossover neckline | Stabilize edge; clean overlap; no rolling. | Binding + clear elastic / bonded trial | 1/8 in |
| C2 | Bodice seams | Curves must be smooth and symmetrical. | Overlock + cover/flatlock | 1/8 in |
| C3 | Waist join | Low-bulk join between bodice, outer skirt and short. | Overlock + understitch as needed | 1/4 in |
| C4 | Flow/pleat panel | Pleats controlled and anchored; drape must not twist. | Specialty stitch / topstitch | 1/8 in |
| C5 | Slit apex | Bar tack or reinforcement patch discreetly. | Reinforced coverstitch | 1/8 in |
| C6 | Hem | Smooth, premium, no tunneling. | Bonded / twin needle | 1/8 in |

# 5. Points of Measure — Base Size M

| POM | Measurement point | Spec (in) | Tol. ± | How measured |
| --- | --- | --- | --- | --- |
| P01 | Bust circumference | 38.0 | 0.5 | 1 in below armhole ×2 |
| P02 | Waist circumference | 31.0 | 0.5 | At waist seam ×2 |
| P03 | Full hip allowance outer layer | 44.0 | 0.6 | At fullest point ×2 |
| P04 | Center back length — above knee | 36.5 | 0.4 | CB neck to hem |
| P05 | Center back length — knee option | 39.5 | 0.4 | CB neck to hem |
| P06 | Front neckline depth | 4.25 | 0.2 | HPS to finished crossover |
| P07 | Armhole circumference | 17.75 | 0.3 | Along seam |
| P08 | Side opening height | 10.0 | 0.3 | Hem to apex |
| P09 | Inner-short inseam | 5.0 | 0.25 | Crotch to leg opening |
| P10 | Pocket opening | 5.5 | 0.25 | Usable finished opening |

# 6. Preliminary Grade Rules

| Measurement | XS→S | S→M | M→L | L→XL | XL→2XL | 2XL→3XL | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bust circumference | 1.5 | 1.5 | 1.5 | 2.0 | 2.0 | 2.0 | Confirm fuller-bust option later |
| Waist circumference | 1.25 | 1.25 | 1.5 | 1.5 | 2.0 | 2.0 | Protect shaping |
| Outer hip allowance | 1.75 | 1.75 | 2.0 | 2.0 | 2.25 | 2.25 | Allow drape over glutes |
| Body length | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | Two approved length variants |
| Opening height | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | Do not overexpose larger sizes |
| Short inseam | 0 | 0 | 0 | 0 | 0 | 0 | Fixed 5 in |

Grade rules are provisional. Final grading must preserve the waist-to-hip relationship, rear coverage, slit position, perforation map and underlayer security on every size.

# 7. Branding, Labels & Packaging

| Element | Placement | Size / method | Color | Notes |
| --- | --- | --- | --- | --- |
| Front emblem | Lower left panel / chest | 12–16 mm | Tonal/gold | Quiet |
| Vertical lockup | Upper center back | 70–90 mm | Tonal/gold | Optional signature level |
| Internal print | Inside upper back | Brand, size, fiber, origin | Tonal | Heat transfer |
| Care label | Inside side seam | Regulatory content | Black/white | Soft/removable |
| Hangtag | Side seam | Story + fit profile + barcode | Black/ivory/gold | Premium uncoated stock |

# 8. Performance & Quality-Control Tests

| Test | Method / movement | Pass criteria | Stage |
| --- | --- | --- | --- |
| Neckline security | Bend, overhead press, bench setup | No gaping or exposure | Proto |
| Panel control | Walk, spin, fan test | Flow panel returns cleanly and does not twist | Proto |
| Slit coverage | Lunge, squat, stairs | Inner short covers fully; outer opening controlled | Proto |
| Machine safety | Simulated proximity to equipment | No loose panel likely to catch during intended use | Risk review |
| Pleat retention | 5 wash cycles if pleat used | Pleat remains defined and even | PP |
| Seam recovery | 20 stretch cycles | No popped stitches or distortion >3% | Lab |

# 9. Sample Review & Revision Log

| Round | Date | Sample type | Issue / observation | Required correction | Status |
| --- | --- | --- | --- | --- | --- |
| Proto 1 |  | Development sample |  |  | Open |
| Proto 2 |  | Revised fit sample |  |  | Pending |
| Size set |  | XS–3XL |  |  | Pending |
| PP sample |  | Pre-production |  |  | Pending |

# 10. Approval Gates

☐ Design architecture approved

☐ Fabric and color approved

☐ Base-size fit approved

☐ Movement and modesty tests passed

☐ Size-set approved

☐ Pre-production sample approved