E W A H

SCULPT  •  FLOW  •  FREEDOM

EWAH-WF-02 ¦ WARRIOR FLOW SHORT SUMMER DRESS ABOVE-THE-KNEE SCULPTED PERFORMANCE DRESS WITH BUILT-IN SHORTS

Approved reference direction: coordinated short Warrior Flow version shown on the right. Preserve the same seam architecture and airflow language as the long Breeze Flow style.

# 1. Style Overview

| FIELD | SPECIFICATION |
| --- | --- |
| Status | Development tech pack draft — not production final |
| Category | Short luxury performance dress / training-to-travel |
| Silhouette | Above-the-knee sculpted dress with dual front-side openings and built-in shorts |
| Primary use | General gym, walking, travel, light lifting, gym-to-life |
| Fit intent | Defined waist, secure bust, fuller hip/glute accommodation, modest movement openings |
| Base size | Curvy M fit model — target body 38 in bust / 31 in waist / 43 in full hip |

# 2. Design Architecture & Technical Callouts

| FIELD | SPECIFICATION |
| --- | --- |
| Neckline | Refined high round neck with mesh insert; secure and supportive. |
| Bodice | Strong sculpted seam architecture derived from Breeze Flow family. |
| Skirt | Above-the-knee outer dress with center solid panel and curved side openings. |
| Underlayer | Opaque fitted short, 5 in inseam, no front seam, right-side phone pocket. |
| Back | Contoured center-back shaping with upper airflow panel and optional vertical EWAH lockup. |
| Hem | Curved front and back hem; rear coverage at least 1.0–1.5 in longer than front on curvy fit model. |
| Non-negotiable | Must not become a generic tennis dress; retain EWAH sculptural lines and controlled modesty. |

# 3. Bill of Materials (BOM)

| COMPONENT | SPECIFICATION | QTY |
| --- | --- | --- |
| Main shell | Matte 4-way stretch nylon/spandex, 220–250 GSM | 1 |
| Ventilation mesh | Fine perforated nylon/spandex, 150–190 GSM | 1 |
| Built-in short | Opaque nylon/spandex interlock, 230–270 GSM | 1 |
| Elastic | Soft 35–40 mm waistband elastic, high recovery | 1 |
| Branding | Tonal or antique-gold heat transfer / silicone | 1 |
| Thread | High-tenacity polyester stretch-compatible thread | As req. |

# 4. Construction & Stitching

| FIELD | SPECIFICATION |
| --- | --- |
| Primary seams | 4-thread overlock or equivalent stretch seam; clean internal finish. |
| Topstitching | Coverstitch or narrow twin-needle where visible; maintain consistent 3 mm visual line. |
| Mesh joins | Stabilize without rigid tape; avoid puckering and abrasion against skin. |
| Slits/openings | Reinforce stress points; clean narrow binding or turned finish. |
| Hem | Bonded or coverstitched according to approved sample; no waviness. |
| Underlayer attachment | Attach at waist seam; avoid bulky seam stack; accessible phone pocket. |
| Brand placement | Front emblem 18–22 mm; vertical back lockup only after exact placement approval. |

# 5. Points of Measure — Base Size M

| POINT OF MEASURE | INCHES | TOLERANCE |
| --- | --- | --- |
| Bust circumference | 38 | ±0.5 |
| Waist circumference | 31 | ±0.5 |
| Full hip allowance — outer | 44 | ±0.75 |
| Center-back length | 34.5 | ±0.5 |
| Front shortest length | 32.5 | ±0.5 |
| Side opening height | 10 | ±0.25 |
| Armhole circumference | 17.5 | ±0.25 |
| Shoulder width | 14.5 | ±0.25 |
| Built-in short inseam | 5 | ±0.25 |
| Short waist relaxed | 29 | ±0.5 |

All measurements are preliminary starting specifications. Final POMs must be set from approved physical pattern and fit sample.

# 6. Preliminary Grade Rules

| FIELD | SPECIFICATION |
| --- | --- |
| Bust | +1.5 in total per size |
| Waist | +1.25 in total per size |
| Full hip / outer ease | +1.75 in total per size |
| Length | +0.25 in per size; petite/tall variants evaluated separately |
| Armhole | +0.25 in circumference per size |
| Built-in short inseam | Hold 5 in unless fit testing requires change |
| Critical rule | Preserve waist-to-hip relationship, slit placement, mesh zoning and rear coverage through grading. |

# 7. Branding, Labels & Packaging

| FIELD | SPECIFICATION |
| --- | --- |
| Primary branding | Small emblem at left hip or lower front; tonal black or antique gold. |
| Back branding | Optional vertical EWAH lockup centered on solid upper-back panel; never printed across open mesh. |
| Internal label | Heat-transfer brand/size label; soft removable care label at side seam. |
| Hangtag | Style code, color, size, fabric composition, coverage level, care QR code. |
| Packaging | Fold with tissue barrier; avoid crushing mesh; individual recycled polybag or premium paper sleeve. |

# 8. Performance & Quality-Control Tests

Deep squat, lunge and deadlift setup

Step-up and overhead reach

Treadmill walking and seated-machine use

Neckline security across movement

Short does not roll or ride excessively

Opacity under stretch and sweat

Rear hem remains covered over fuller glutes

Side openings do not expose beyond approved coverage

# 9. Sample Review & Revision Log

| ROUND | DATE | SAMPLE TYPE | ISSUE / OBSERVATION | ACTION | STATUS |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Proto 1 |  |  | Open |
| 2 |  | Fit sample |  |  | Open |
| 3 |  | Size set |  |  | Open |
| 4 |  | Pre-production |  |  | Open |

# 10. Approval Gates

☐ Design architecture approved

☐ Fabric and color approved

☐ Base-size fit approved

☐ Movement and modesty tests passed

☐ Size-set approved

☐ Pre-production sample approved